| College of Computer Science Northeastern University CSG150: Fundamentals of Computer Networking | Fall 2003 December 1st, 2003 |
| --- | --- |

# Problem Set 6

# (due before December 12th)

[20 points] Problem 5.4, textbook page 480.

[20 points] Problem 5.1, textbook page 479 (Apply only Prim-Dijkstra algorithm).

[20 points] Problem 5.3, textbook page 479.

[20 points] OSPF standard:

- Find the OSPF (version 2)  IETF RFC  and provide the http link.

- What is the IP protocol value for OSPF?

- What is the difference between OSPF and a protocol like RIP or BGP?

- Explain the fields of an OSPF Link State Advertisement (LSA) packet.

- Draw a simple network topology and provide the details of an LSA packet.

[20 points] Fair Queuing (wait until next lecture to solve it):

Three connections A, B, C, of equal weight share the same link.

t = 0: packets arrive for the connections with length 1, 2, 3

t = 5: one packet of size 3 arrives for connection A,

t = 7: one packet of size 1 arrives for connection C.

- Draw the round number R(t) as a function of time for the ideal GPS (Max-Min fairness).

- Compute the finish number for each packet F(A/B/C, i, t) when using weighted fair queuing (WFQ).

- In what order would the WFQ schedule the transmission of the packets?